AWS VPC Traffic Mirroring

What is AWS VPC Traffic Mirroring?

AWS VPC Traffic Mirroring is a feature that allows you to capture and inspect network traffic from an Elastic Network Interface (ENI) in your Virtual Private Cloud (VPC). This traffic is then sent to a monitoring or security tool for analysis. It works like a virtual tap that copies network packets in real time.

Example of VPC Traffic Mirroring

Imagine you have an EC2 instance running a web application. If you want to analyze all incoming and outgoing network traffic for security or performance monitoring, you can enable VPC Traffic Mirroring on the ENI of that instance. The mirrored traffic can be sent to a network analysis tool such as AWS Network Packet Broker or a third-party security solution.

Use of AWS VPC Traffic Mirroring

VPC Traffic Mirroring helps in:

- Deep Packet Inspection (DPI): Helps security teams inspect individual network packets to detect potential threats.

- Intrusion Detection and Prevention (IDS/IPS): Enhances security by identifying and preventing malicious activities in the network.

- Troubleshooting Network Issues: Helps diagnose connectivity issues by analyzing the network traffic in real time.

- Performance Monitoring: Ensures optimal network performance by identifying latency or bottlenecks.

- Compliance and Auditing: Helps organizations meet security and compliance requirements by logging and analyzing traffic.

Where Can We Use AWS VPC Traffic Mirroring?

VPC Traffic Mirroring is useful in several scenarios:

- Security Monitoring: To detect and prevent cyber threats, malware, or data breaches.

- Forensic Analysis: To analyze past network activities and investigate security incidents.

- Network Performance Optimization: To identify performance issues and optimize workloads.

- Regulatory Compliance: To ensure that network security policies meet industry standards such as GDPR, HIPAA, and PCI DSS.

- Cloud Migration Analysis: To monitor traffic patterns before migrating workloads to the cloud.

Scenarios Explaining the Use of VPC Traffic Mirroring

- Detecting a DDoS Attack:

  - A company experiences sudden spikes in network traffic, slowing down its application. Using VPC Traffic Mirroring, security analysts inspect the traffic and detect a Distributed Denial-of-Service (DDoS) attack. The company then blocks malicious IPs using AWS WAF.

- Identifying Unauthorized Access:

  - A financial institution suspects unauthorized access to its internal resources. By mirroring traffic from critical EC2 instances, the security team discovers that an attacker is attempting to access confidential data using compromised credentials. The team takes immediate action to revoke access and prevent a breach.

- Troubleshooting Connectivity Issues:

  - A cloud-based e-commerce website is facing intermittent connectivity issues. The network team enables Traffic Mirroring to capture real-time traffic and identify packet drops caused by a misconfigured security group rule. They correct the configuration, restoring connectivity.

- Ensuring Compliance for PCI DSS regulations:

  - A payment processing company must comply with PCI DSS regulations, requiring all traffic to be monitored for potential security breaches. They use VPC Traffic Mirroring to capture and analyze traffic for any unauthorized access attempts or data leaks.

The PCI DSS is maintained by the PCI Security Standards Council (PCI SSC), which was founded by major payment brands such as Visa, MasterCard, American Express, Discover, and JCB.

PCI DSS isn’t a law in itself—it’s a contractual obligation between businesses and payment card companies. However, many governments enforce it indirectly because banks and payment processors require compliance before allowing you to handle card payments.

- Monitoring Application Performance:

  - A SaaS provider wants to optimize application response times. By mirroring traffic, the DevOps team detects latency in database queries due to high network congestion. They optimize queries and upgrade network bandwidth, improving performance.

Step-by-Step Lab to Configure AWS VPC Traffic Mirroring (2025)

Step 1: Prerequisites

- AWS Account with appropriate IAM permissions.

- A VPC with at least two EC2 instances:

  - Source EC2 Instance: The instance whose traffic you want to mirror.

  - Target EC2 Instance: The instance that will receive and analyze mirrored traffic.

- Ensure both instances are in the same or peered VPC or connected using PrivateLink.

Step 2: Enable VPC Traffic Mirroring

1. Create a Traffic Mirror Target

- Open the AWS Management Console and navigate to VPC Dashboard.

- Click on Traffic Mirroring > Mirror Targets.

- Click Create Traffic Mirror Target.

- Select the ENI (Elastic Network Interface) of the target EC2 instance.

- Name the target and click Create.

2. Create a Traffic Mirror Filter

- Go to Traffic Mirroring > Mirror Filters.

- Click Create Traffic Mirror Filter.

- Define inbound and outbound rules (e.g., allow all traffic or specific ports like SSH and HTTP).

- Click Create.

3. Create a Traffic Mirror Session

- Navigate to Traffic Mirror Sessions and click Create Traffic Mirror Session.

- Select the Source ENI (the instance whose traffic will be mirrored).

- Select the Mirror Target (created in Step 1).

- Select the Mirror Filter (created in Step 2).

- Set Packet Length and Session Number (lower numbers have higher priority).

- Click Create.

Important Note:

1. Packet Length

This is how much of each network packet AWS will copy and send to your traffic mirror target.

- If you set PacketLength = 0 or null → AWS mirrors the entire packet (full payload + headers).

- If you set a positive value (like 128, 256, etc.) → AWS captures only the first N bytes of the packet. The rest is truncated.

Best practice:

- If you are doing full deep packet inspection (DPI) or need complete payloads (security analysis, forensics, intrusion detection), set it to 0.

- If you just need metadata or header analysis (IP/TCP headers, basic NetFlow-like analysis), something like 128 or 256 bytes is enough — this reduces costs and processing overhead.

- Many security appliances recommend 128–150 bytes for header-based monitoring to keep mirror traffic smaller.

2. Session Number

The Session Number decides the priority when you have multiple traffic mirror sessions for the same ENI (Elastic Network Interface).

- Range: 1 to 32766

- Lower number = higher priority (1 is the highest)

- If two sessions overlap (mirror the same traffic), the lowest session number wins.

- If priorities are the same, AWS uses the most specific traffic filter.

Best practice:

- If you have only one mirror session per ENI, set it to 1.

- If you have multiple sessions (e.g., one for IDS, one for logging), give the most important session a lower number (1, 2…) and the others higher numbers (10, 20…).

3. VNI (VXLAN Network Identifier)

AWS sends mirrored traffic over VXLAN encapsulation to your target.
VNI is the VXLAN Network Identifier — a 24-bit value (range 1–16,777,215) used to tag the mirrored traffic.

- The VNI is included in the VXLAN header so your target system can distinguish traffic from different sessions.

- This is useful if:

  - You send mirrored traffic from multiple sessions to the same target

  - Your analysis tool needs to separate traffic streams

- If you don’t care about multiple separation, you can just keep the default AWS-assigned value.

Best practice:

- If one mirror target is receiving traffic from multiple sources/sessions, set unique VNI per session so your tool can tell them apart.

- If you have only one session, you can leave VNI as default (AWS picks it).

Step 3: Test and Verify Traffic Mirroring

1. Generate Traffic on the Source EC2 Instance

Run the following commands on the source instance to create network activity:

ping -c 5 google.com

curl -I http://example.com

2. Capture Mirrored Traffic on the Target EC2 Instance

Install and use tcpdump to verify traffic mirroring:

sudo yum install tcpdump -y

ip a

sudo tcpdump -i enX0 -nn

or

sudo tcpdump -i any -nn

Limit the Number of Captured Traffic

sudo tcpdump -i enX0 -nn -c 50

Filter Traffic by Port

sudo tcpdump -i enX0 -nn port 22

Save Captured Packets to a File for Later Analysis

sudo tcpdump -i enX0 -nn -w traffic_capture.pcap

Then analyze the file using Wireshark:

wireshark traffic_capture.pcap

You should see mirrored packets from the source instance.

Step 4: Analyze Mirrored Traffic

- Use Wireshark or Zeek (Bro IDS) for deeper packet inspection.

- Send mirrored traffic to AWS tools like Amazon Kinesis, AWS Network Firewall, or a third-party security solution.

- Set up an AWS Lambda function to log or analyze traffic patterns.

Output Analysis:

Here’s your first line:sss

13:27:49.750786 IP 13.233.177.3.27916 > 172.31.10.228.22: Flags [.], ack 228, win 443, options [nop,nop,TS val 2261632359 ecr 2455864069], length 0

1. Timestamp

13:27:49.750786
This is the time the packet was captured (HH:MM:SS.microseconds).

2. Protocol

IP
The packet is using IPv4 (tcpdump will also show IP6 for IPv6).

3. Source and Destination

13.233.177.3.27916 > 172.31.10.228.22

- Source IP: 13.233.177.3 (likely a public AWS address — maybe your SSH client or another machine)

- Source Port: 27916 (ephemeral port used by the client)

- Destination IP: 172.31.10.228 (private AWS EC2 IP)

- Destination Port: 22 (SSH service)

So, this packet is going from the client to the EC2’s SSH port.

4. TCP Flags

Flags [.]

- . means ACK only (no data is sent, just acknowledging receipt of data).
Common TCP flags:

- S = SYN (connection start)

- F = FIN (connection end)

- P = PSH (push data immediately)

- . = ACK

Here, the client is acknowledging it received bytes from the server.

5. TCP Sequence / Acknowledgement

ack 228

- The client is saying: "I have received all data up to byte number 227, and now I’m expecting byte 228 from you."

6. TCP Window Size

win 443

- The client's TCP receive window is 443 bytes — meaning it can currently accept 443 more bytes before it must acknowledge again.

7. TCP Options

options [nop,nop,TS val 2261632359 ecr 2455864069]

- nop = No Operation (padding in TCP options)

- TS val = Sender's timestamp (used for RTT calculation)

- ecr = Echo reply (timestamp from the other side, for measuring round-trip time)

8. Payload Length

length 0

- No data payload in this packet — it’s purely an ACK.

Second Line:

13:27:49.809639 IP 172.31.10.228.22 > 13.233.177.3.27916: Flags [P.], seq 228:600, ack 1, win 482, options [nop,nop,TS val 2455864129 ecr 2261632359], length 372

This is the reverse direction — from your EC2 (server) to the client.

Breakdown:

- Flags [P.] → Push + ACK

  - Push (P) means “send the data to the application immediately” (SSH here).

- seq 228:600 → This packet carries bytes 228 to 599 (372 bytes total).

- ack 1 → The server is acknowledging it has received the first byte from the client.

- win 482 → TCP window size for the server.

- length 372 → This packet contains 372 bytes of SSH data.